SusX Pothole Detection System Code Logic Explained in Plain Language ESP32 + MPU6050 + NEO-6M GPS + Firebase ¦ v2.0

# What Does This System Do?

This system is a smart pothole detector mounted in a vehicle. It uses:

- An MPU6050 sensor (measures movement and tilt)

- A GPS module (knows where you are and how fast you are going)

- An ESP32 microcontroller (the brain that runs everything)

- Firebase cloud database (stores pothole reports online)

- SPIFFS flash memory (stores reports locally when WiFi is unavailable)

The core problem it solves: older systems simply checked "did the car shake hard enough?" and reported it as a pothole. This caused massive false alarms from speed breakers, braking, and engine vibration. This v2.0 system uses a 4-phase pipeline that thinks about the shake, not just how big it was.

Simple Analogy: Imagine a doctor who does not just check your temperature — they also check your pulse, breathing rate, and how long you have had a fever before diagnosing you. This system does the same for road bumps.

# The 4-Phase Detection Pipeline

The firmware processes sensor data through four sequential phases every time the vehicle is running. Think of it as an assembly line — data goes in one end and a verdict (pothole / speed breaker / nothing) comes out the other.

| Phase | Name | What It Does |
| --- | --- | --- |
| Phase 1 | Calibration | Measures how the sensor is tilted before driving. Sets the gravity baseline. |
| Phase 2 | Filtering | Reads sensor 100 times per second and removes noise (engine vibration, gravity). |
| Phase 3 | Triggering | Watches for a sudden jerk. If found, captures data for the next 300 ms. |
| Phase 4 | Classification | Analyses the captured data using 4 rules to decide what happened. |

# Phase 1 — Calibration

## The Problem: Sensor Tilt

The MPU6050 sensor measures acceleration in 3 directions: X (forward/backward), Y (left/right), Z (up/down). If the sensor is perfectly flat, gravity pulls exactly downward — all 9.81 m/s² shows on the Z axis alone.

But sensors are mounted on dashboards, which are tilted. If the sensor is tilted 15 degrees, some of that gravity "bleeds" into the X and Y axes. This would ruin all the later math that checks axis ratios.

Simple Example: Put a spirit level on your dashboard. It will never read perfectly flat. The calibration step measures that tilt precisely and corrects for it mathematically.

## What the Code Does

On startup, while the vehicle is still, the ESP32 reads the sensor 200 times over 2 seconds. It averages all the readings to get the "true resting baseline" for each axis:

Code Snippet (simplified): gravityOffsetZ = average of 200 Z readings gravityOffsetX = average of 200 X readings gravityOffsetY = average of 200 Y readings

From this point on, every reading has these offsets subtracted. So if the sensor sees Z = 10.2 m/s² and the calibrated baseline is 9.81 m/s², the dynamic shock is just 0.39 m/s². This is like zeroing a kitchen scale before weighing something.

# Phase 2 — Signal Filtering

## Two Noise Problems

After calibration, the sensor still sees two types of unwanted noise:

- Engine vibration — a car engine at 3,000 RPM creates vibrations around 50-100 Hz. These are too fast and small to be a pothole, but they show up in raw data.

- Micro-texture road noise — normal asphalt creates constant low-level buzz in all axes.

If we calculate jerk (the rate of change of acceleration) directly on noisy data, these vibrations would cause constant false alarms. We need to clean the signal first.

## The IIR Low-Pass Filter

The firmware applies a digital low-pass filter — this lets slow changes through but blocks fast, high-frequency vibrations. Think of it like a smoothing average, but smarter.

Simple Analogy: Imagine you are tracking daily temperature. A single-day reading might be wildly wrong. Averaging the last 5 days gives a smooth, reliable trend. The low-pass filter does this for acceleration data — 100 times per second.

The formula is: new_filtered = 0.486 x current_reading + 0.514 x previous_filtered

This means: 48.6% of the new reading + 51.4% of the old smoothed value. This lets genuine shocks through (which are big) while ignoring tiny engine vibrations (which are small and fast).

## 100 Hz Sampling — The Ring Buffer

The sensor is read exactly 100 times per second (every 10 milliseconds). Each filtered reading is stored in a circular ring buffer — an array of 100 slots that acts like a conveyor belt. When it fills up, new data overwrites the oldest data.

Simple Example: Imagine a CCTV camera that only keeps the last 60 seconds of footage and automatically deletes older recordings. The ring buffer keeps exactly the last 1 second of sensor data at all times.

# Phase 3 — Dynamic Triggering

## What is Jerk?

Jerk is the rate of change of acceleration — how quickly the acceleration is increasing or decreasing. It is measured in m/s³.

This is the single most powerful feature for telling potholes from speed breakers:

| Road Event | Jerk Value | Why |
| --- | --- | --- |
| Pothole | Very HIGH (>14 m/s³) | Sharp rigid edge causes near-instant acceleration change |
| Speed Breaker | Low-Moderate (<10 m/s³) | Smooth curved surface = gradual acceleration change |
| Braking | Moderate, sustained | Slow deceleration over 1-2 seconds on the X axis |
| Engine vibration | Near zero after filtering | Removed by the low-pass filter in Phase 2 |

## Speed-Adaptive Threshold

A fixed jerk threshold fails in the real world. Physics tells us: the faster you go over a bump, the bigger the jerk — even if the bump itself is tiny. A shallow crack at 90 km/h produces more jerk than a deep pothole at 10 km/h.

The solution is a threshold that grows with speed:

Formula: Trigger Threshold = 8.0 + (0.15 x speed in km/h) At 20 km/h: threshold = 8.0 + 3.0 = 11.0 m/s³ At 60 km/h: threshold = 8.0 + 9.0 = 17.0 m/s³ At 100 km/h: threshold = 8.0 + 15.0 = 23.0 m/s³

This keeps the system sensitive in slow residential zones while suppressing false positives on highways where even normal road joins create large jerk values.

## Speed Gate

If the vehicle is going slower than 5 km/h, the system completely ignores all acceleration data. At very low speeds, the suspension absorbs shocks slowly and jerk values are meaningless. This also prevents false reports from door slams, engine starts, and parking maneuvers.

Simple Example: Imagine a speed camera that only activates above 5 km/h. Below that, it ignores all cars — including your own car parking. The speed gate works exactly the same way.

## The 300ms Capture Window

When the jerk threshold is exceeded, the system does NOT immediately classify the event. Instead it:

- Transitions from IDLE to EVENT_CAPTURE state

- Counts down 30 more sensor samples (30 x 10ms = 300ms)

- Waits for the full physical impact to play out and the suspension to settle

- Then transitions to CLASSIFYING and analyses the full 300ms of buffered data

Simple Analogy: Think of a cricket umpire who does not raise the finger immediately when the ball hits the pad. They watch the whole delivery, see where the batsman was standing, and only then make a decision. The 300ms window is that 'watching time'.

# Phase 4 — Multi-Axis Classification

## Why Multiple Axes?

The original code only used the Z axis (up/down). The critical insight from research is that potholes and speed breakers have completely different signatures on ALL three axes and the gyroscope:

| Axis / Sensor | Speed Breaker Signature | Pothole Signature |
| --- | --- | --- |
| Z axis (up/down) | Large smooth positive peak | Sharp negative dip, then spike |
| Y axis (left/right) | Near zero — both wheels hit equally | Large impulse — only one wheel drops |
| X axis (forward) | Moderate negative — car decelerates | Small — no braking involved |
| Gyro PITCH (Y-rotation) | Very HIGH — front lifts symmetrically | Low — no full-front rotation |
| Gyro ROLL (X-rotation) | Near zero — both sides equal | Very HIGH — one side drops, car rocks |

## The Z/Y Ratio — The Key Test

The most powerful single number is the ratio of peak Z acceleration to peak Y acceleration:

- HIGH Z/Y ratio (> 3.0): The vertical shock was big but lateral movement was small. This means both sides of the car hit something at the same time — a speed breaker or expansion joint.

- LOW Z/Y ratio (< 3.0): The vertical shock was large AND there was significant sideways movement. This means only one wheel dropped — a pothole.

Simple Example: Imagine picking up a suitcase by both handles (symmetric = speed breaker) vs. only one handle (asymmetric = pothole). With one handle, the bag tilts sideways — that lateral tilt is the Y-axis signal.

## Rule 1 — Braking Rejection

Hard braking produces a large sustained negative value on the X axis (forward deceleration) that lasts for hundreds of milliseconds. A pothole does NOT cause this. If the code sees:

- Average X acceleration below -5.0 m/s² across the event window, AND

- No "dip then spike" pattern on the Z axis

... it classifies the event as Braking and discards it.

Simple Example: You are on a bus and it brakes sharply. You lurch forward (X axis). The road is perfectly flat. The system sees the X-axis lurch, no Y-axis rock, no Z dip-spike, and correctly ignores it.

## Rule 2 — Speed Breaker Rejection

Speed breakers span the full width of the road, so both front wheels hit at the same time. This produces:

- High PITCH rate (the front of the car rotates upward)

- Near-zero ROLL rate (both sides lift equally, no rocking)

- High Z/Y ratio (big vertical movement, minimal lateral movement)

If all three conditions are true, the event is classified as SpeedBreaker and discarded.

Simple Example: Think of a see-saw. If two children of equal weight sit on both ends and someone pushes the middle, it goes up evenly (speed breaker = high pitch, no roll). If only one child sits down hard, the whole see-saw tilts sideways (pothole = high roll).

## Rule 3 — Pothole Confirmation

A pothole is confirmed only when ALL of these are true:

- Peak ROLL rate > 30 degrees/second (the car rocked sideways)

- Z/Y ratio < 3.0 (lateral movement confirms asymmetric impact)

- Z axis shows a negative dip immediately before a positive spike (the tire dropped into the hole, then hit the far edge)

Dip-Spike Pattern Explained: When a tyre hits a pothole: 1) The front wheel crosses the edge and drops into the void — the car briefly feels lighter (negative Z). 2) Milliseconds later, the tyre slams into the far wall of the pothole — violent positive Z spike. This 'dip then spike' sequence is the fingerprint of a real pothole.

## Rule 4 — Default

If none of the above rules fire, the event is classified as Normal Vibration and silently discarded. This covers cobblestones, railroad crossings, rough tarmac, and any other event that does not match the specific pothole signature.

# Severity Grading

Once a pothole is confirmed, the system grades its severity based on the peak vertical acceleration (Z axis) recorded during the event:

| Severity | Threshold | Real-world meaning | Buzzer |
| --- | --- | --- | --- |
| Minor | < 3g (29.4 m/s²) | Small depression, low risk | 1 beep |
| Moderate | 3g to 5g (29.4-49 m/s²) | Significant pothole, report to council | 2 beeps |
| Severe | > 5g (49 m/s²) | Dangerous — risk of tyre and suspension damage | 3 fast beeps |

# Data Flow — From Sensor to Cloud

## Step-by-Step Journey of a Pothole Report

| Step | Where | What Happens |
| --- | --- | --- |
| 1 | MPU6050 Sensor | Measures acceleration (m/s²) and gyroscope (deg/s) 100 times per second. |
| 2 | Phase 2 Filter | Gravity removed. IIR low-pass filter applied. Stored in ring buffer. |
| 3 | Phase 3 Trigger | Jerk computed. If jerk > dynamic threshold AND speed > 5 km/h, capture starts. |
| 4 | Phase 4 Classify | 4 rules applied to 300ms of buffered data. Verdict reached. |
| 5 | Severity Grade | Peak Z-axis used to rate Minor / Moderate / Severe. |
| 6 | Buzzer | 1, 2, or 3 beeps played to alert the driver. |
| 7a | Firebase (online) | Full JSON report uploaded: lat, lng, severity, g-force, speed, timestamp. |
| 7b | SPIFFS (offline) | If WiFi is down, CSV line saved to flash memory. |
| 8 | Upload on reconnect | When WiFi restores, all buffered SPIFFS reports are uploaded and the file is cleared. |

## Firebase Payload Example

Each confirmed pothole uploads a structured JSON document to Firebase Realtime Database:

Example Firebase Record: /potholes/report_7 lat: 19.076245 lng: 72.877812 type: "Pothole" severity: "Moderate" g_force: 3.42 speed_kmh: 34.5 timestamp_ms: 183450 confirmed: false pass_count: 1

The confirmed field starts as false. In a full deployment, a cloud function would set it to true once 4 or more different vehicles report a pothole at the same GPS location — this is based on the MIT Pothole Patrol (P²) research method, which uses crowdsourcing to eliminate one-off sensor glitches.

# Tunable Parameters

All key values are defined as constants at the top of the code (Section 1). You can adjust these without touching any detection logic:

| Parameter | Default | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- |
| MIN_SPEED_KMH | 5.0 | Ignores more slow-speed events | More false positives when parking |
| BASE_JERK_THRESH | 8.0 m/s³ | Fewer detections overall | More false positives |
| SPEED_JERK_COEFF | 0.15 | Suppresses more highway events | May miss high-speed potholes |
| ROLL_THRESH | 30 °/s | Only catches severe swerving | May confuse gentle curves as potholes |
| PITCH_THRESH | 35 °/s | Fewer speed breaker rejections | More speed breakers flagged as potholes |
| ZY_HIGH_LIMIT | 3.0 | Harder to confirm potholes | More symmetric events misclassified |
| LPF_ALPHA | 0.486 | Less smoothing (noisier) | More smoothing (slower response) |
| DEBOUNCE_MS | 2500 ms | Longer gap between reports | Risk of double-reporting same hole |

Tuning Tip: Start by adjusting ROLL_THRESH and PITCH_THRESH first — these are the most vehicle-dependent values. A heavy SUV will produce lower roll rates than a small hatchback for the same pothole, so these may need to be reduced for SUVs.

# v1.0 vs v2.0 Comparison

| Feature | v1.0 (Original) | v2.0 (This System) |
| --- | --- | --- |
| Detection method | Single Z-axis threshold | 4-phase multi-axis pipeline |
| Speed breaker detection | Not possible | Rejected by pitch rate + Z/Y ratio |
| Braking false positives | Common | Rejected by sustained X-axis check |
| Engine vibration | Caused false triggers | Removed by IIR Low-Pass Filter |
| Sensor tilt | Ignored (assumed flat) | Corrected by stationary calibration |
| Speed awareness | None | Dynamic threshold scales with speed |
| Slow-speed gating | None | Detection disabled below 5 km/h |
| Jerk analysis | None | Primary trigger feature |
| Gyroscope usage | None | Roll vs. Pitch ratio for classification |
| Firebase payload | lat, lng, severity, g-force | Adds type, speed, structured pass_count |
| Offline buffer format | Basic text file | Validated CSV with type field |
| Expected accuracy | ~50-60% (high false positives) | ~85-92% (based on research benchmarks) |

# Summary

Here is the complete logic in one sentence:

Sample 100 times/sec → Remove gravity and noise → Watch for sudden jerk → Capture 300ms → Check roll, pitch, Z/Y ratio, and dip-spike pattern → Report only confirmed potholes with GPS + severity

Every step exists to answer one question: was that jolt actually a pothole, or something we should ignore? The multi-axis evidence makes the answer reliable.

SusX Pothole Detection System v2.0  —  Technical Documentation